# UNIT TITLE: ICT SYSTEM SUPPORT

UNIT CODE:

Relationship to Occupational Standards

This unit addresses the unit of competency: Perform ICT system support

Diploma certificate shall be classified into Distinction, Credit I, Credit II and Pass

| Percentage Marks | Class |
| --- | --- |
| Distinction | 70 and above |
| Credit I | 65-69 |
| Credit II | 60-64 |
| Pass | 50-59 |
| Fail | Below 50 |

Unit Description:

This unit describes the competencies required to perform ICT infrastructure support. It involves identification and documentation of ICT infrastructure, evaluation of the state of performance and possible causes of failure, diagnosing and fixing of the problems, testing of performance and user training.

Summary of Learning Outcomes:

By the end of the unit, the trainee should be able to:

- Identify and Document ICT infrastructure equipment

- Evaluate performance of ICT components Diagnose and fix problems

- Diagnose and fix ICT related problems

- Perform User training

Learning Outcomes, Content and Suggested Assessment Methods

| Learning Outcome | Content | Suggested Assessment Methods |
| --- | --- | --- |
| 1. Identify and Document ICT infrastructure equipment | Definition of ICT infrastructure Components of ICT Infrastructure ICT Infrastructure specifications Types of ICT infrastructure  Computer hardware platforms  Operating system platforms  Enterprise and other software applications  Data management and storage  Networking and  telecommunications platforms  Internet platforms  End users Safety precautions of ICT Infrastructure | - Practical exercises - Oral questioning - Learner portfolio of evidence |

1

|  | Documentation of Infrastructure assets and their operational and service status |  |
| --- | --- | --- |
| 2 Evaluate performance of ICT components | Define troubleshooting Possible causes of failure  Unstable power  Malfunctioning  Mechanical faults Environmental factors  Natural disasters  Dust  Ventilation User factors  Malicious damage  Accidents  Lack of maintenance | - Observation - Practical - Projects |
| 3. Diagnose and fix ICT related problems | Define Diagnostic terms Identify diagnostic and repair tools and their functions  Tools to diagnose and fix the problems. Hardware related problems Software related problems Internet/network related problems User related problems Role of ICT Policies in organizations | - Practical exercises - Oral questioning - Written tests - Learner portfolio of evidence. |
| 4. Perform User training |  Meaning of user training  Importance of user training  Implement end user training plan | - Practical exercises - Oral questioning - Learner portfolio of evidence. - Observation |

Suggested Methods of Delivery

- Presentations and practical demonstrations by trainer;

- Guided learner activities and research to develop underpinning knowledge;

- Supervised activities and projects in a workshop;

The delivery may also be supplemented and enhanced by the following, if the opportunity allows:

- Visiting lecturer/trainer from the ICT sector;

- Industrial visits.

Equipment

25 Computers

1 Printer

1 Server

1 Scanner

Network components

Digital instructional material including DVDs and CDs;

Trunking

100 Cable ties

Power

Tools

Comprehensive set of hand tools for the

Recommended Resources for 25 Trainees

2

Reference materials

Manufacturers manuals

5 Network cabinets

# Introduction

Definition of ICT System Support:

ICT system support refers to the provision of assistance, maintenance, and troubleshooting services for information and communication technology systems. These systems can include hardware, software, networks, and associated components.

# Role of ICT System Support:

 Ensure the smooth functioning of ICT systems.

 Address technical issues and problems promptly.

 Assist users with technology-related questions and concerns.

 Keep systems up to date with software updates and security patches.

 Monitor network and system performance.

 Plan for system upgrades and improvements.

 Backup and data recovery.

 Ensure data security and privacy.

# ICT INFRASTRUCTURE EQUIPMENT

Definition of ICT infrastructure

 IT infrastructure is defined as a combined set of hardware, software, networks, facilities, etc. (including all of the information technology related equipment) used to develop, test, deliver, monitor, control, or support IT services.

 Information communication technology (ICT) infrastructure are the components required to operate and manage enterprise IT environments. IT infrastructure can be deployed within a cloud computing system, or within an organization's own facilities.

These components include hardware, software, networking components, an operating system (OS), and data storage, all of which are used to deliver IT services and solutions. IT infrastructure products are available as downloadable software applications that run on top of existing IT resources or as online solutions offered by service providers.

# Components of ICT Infrastructure

There are three main components of ICT infrastructure

  Hardware

  Software

  Networking

# Hardware

Hardware includes servers, datacenters, personal computers, routers, switches, and other equipment.

3

# Software

Software refers to the applications used by the business, such as web servers, content management systems, and the OS. The OS is responsible for managing system resources and hardware, and makes the connections between all of your software and the physical resources that do the work.

# Networking

Interconnected network components enable network operations, management, and communication between internal and external systems. The network consists of internet connectivity, network enablement, firewalls and security, as well as hardware like routers, switches, and cables.

# Other components of IT Infrastructure and Examples of Infrastructure Technology Computer Hardware Platforms

These are computers and server machines in your network. Servers allow users to share information and the necessary files required to keep the company running. Some processors allow limited programmability like most video accelerators while in others, you can program them fully. Some manufacturers of quality computer hardware and servers include IBM, Apple, HP, and DELL, among others.

# Enterprise and Software Applications

Enterprise application is a large-scale software that helps solve the problems of the entire organization. They include software such as middleware, Oracle, and Peoplesoft, it is a type of os that allows different parts of a software program to run concurrently with other applications in the companies used to link all the other applications also fall into this category.

# Operating System Platforms

Operating System Platforms are a type of OS that allows different parts of a software program to run concurrently. Every computer must have at least one OS in order to function. OS platforms such as Windows, macOS, and servers like Linux are used by the employees and other personnel in the company for communication, storage, and access of data.

# Data Storage and Management

Data management is handled by software and stored in storage devices. Storage management comprised of disks that are easy to reformat when needed. The number of disks is added as information increases. Additionally, data is stored in different locations to avoid total loss in case of a breakdown or disaster.

# Network and Telecommunication Platforms

Networking platforms include Windows server OS, Linux, Unix, among others. Telecommunication platforms are provided by telecommunication service companies that provide data connectivity, internet access, voice, and wired area network.

# Internet Platforms

Major components of the internet relate to and sometimes overlap with network infrastructure and hardware and software platforms. In a company, infrastructure related to the internet includes the hardware, software, and facilities that support web hosting, maintenance of websites, and web application tools together with intranets and extranets. By understanding the core concepts of the components of the internet, companies can ensure their systems are secure and safe from outside intrusions.

4

# Consulting and System Integration Services

These components of information systems are used to integrate the legacy system with modern infrastructure. Software integration is ensuring that new technology works with the old one also called legacy system. Companies continue using their old system since replacing them is very expensive.

# How do the Components of IT Infrastructure Work?

 Facilities

Networking devices, servers, and data centres are housed in facilities or physical plants. It also covers the network cabling that connects components of an IT system in office buildings.

 Network

Switches, routers, hubs, and servers make up a network. Local area networks (LANs) use switches to connect network devices such as routers, servers, and other switches. Routers allow devices on separate LANs to communicate and exchange packets. Hubs bring together several networking devices to function as a single unit.

 Server

A server is a critical piece of hardware for any company IT infrastructure. Servers are machines that allow numerous users to share resources and access them.

#  Data Centre/Server Room

Server rooms, often known as data centres, are where an organization's various servers are housed. Most networks are built around data centres.

# Benefits of IT Infrastructure

Business high-performance storage, a low-latency network, security, efficient wide area network (WAN), virtualization, and zero downtime will all be provided by the ideal IT infrastructure.

#  High-performance Storage

It back up and store data, as well as providing a data recovery solution in the event of a disaster.

#  Low-latency Networks

To reduce data flow delays, deploy enterprise-level infrastructure components.

 Secure

Systems that manage information access and data availability are included in infrastructures. It can help protect a company from data breaches and cyberattacks, ensuring that customers' trust is maintained.

#  Virtualization

It enables faster server provisioning, higher uptime, better disaster recovery, and energy savings.

#  Zero Downtime

It strives to minimise business disruptions and eliminate system downtime in order to keep costs low and revenues high.

# ICT Infrastructure specifications

ICT (Information and Communication Technology) infrastructure specifications are detailed descriptions of the requirements, standards, and configurations that govern the design, implementation, and management of an organization's technology infrastructure. These specifications are essential for ensuring that ICT systems operate efficiently, securely, and in alignment with business objectives. Here are key points to consider when documenting ICT infrastructure specifications:

# Purpose of Specifications:

ICT infrastructure specifications define the purpose, goals, and scope of the infrastructure project. This includes outlining the specific needs and objectives that the infrastructure should address.

5

# Technical Requirements:

Detail the technical requirements for hardware, software, networking equipment, and other components of the infrastructure.

Specify performance metrics, capacity, compatibility, and scalability requirements.

# Standards and Protocols:

Specify the technology standards, protocols, and industry best practices that must be followed. Include details about data communication protocols, security standards, and compliance requirements.

# Infrastructure Architecture:

Describe the overall architecture of the infrastructure, including network topology, data center layout, and cloud integration. Specify redundancy and failover configurations for high availability.

# Hardware Specifications:

Detail the specifications for servers, storage devices, switches, routers, firewalls, and other hardware components. Include information on power requirements, rack space, and cooling needs.

# Software Requirements:

List the required operating systems, software applications, and middleware. Specify licensing agreements, version compatibility, and update policies.

# Security Measures:

Define security requirements, including access controls, encryption standards, firewall rules, and intrusion detection/prevention systems. Specify data protection measures and disaster recovery plans.

# Network Infrastructure:

Provide specifications for network design, including IP addressing schemes, VLAN configurations, and bandwidth requirements. Include network diagrams and routing plans.

# Data Storage and Backup:

Detail data storage requirements, such as storage capacity, data retention policies, and backup strategies. Specify backup and recovery procedures.

# Monitoring and Management:

Describe monitoring tools, management software, and procedures for system health checks and performance monitoring. Define alert thresholds and escalation procedures.

# Compliance and Regulations:

Ensure that the infrastructure specifications comply with relevant industry regulations (e.g., GDPR, HIPAA) and legal requirements. Specify auditing and reporting mechanisms.

# Budget and Resources:

Outline the estimated budget for the infrastructure project, including costs for hardware, software, labor, and ongoing maintenance. Identify the human resources and skill sets required for implementation and support.

# Timeline and Milestones:

6

Create a project timeline with key milestones and deadlines for infrastructure deployment. Define the sequence of tasks and dependencies.

# Testing and Validation:

Specify the testing procedures, including performance testing, security testing, and user acceptance testing. Document criteria for successful validation.

# Documentation and Knowledge Transfer:

Include requirements for comprehensive documentation of the infrastructure, including system diagrams, configuration guides, and user manuals. Plan for knowledge transfer to IT staff and end-users.

# Change Management:

Describe change control processes for making modifications or updates to the infrastructure. Specify how changes will be tested and approved.

# Vendor and Supplier Agreements:

Document agreements with vendors and suppliers, including service level agreements (SLAs), warranties, and support contracts. Specify service response times and escalation procedures.

# Disaster Recovery and Business Continuity:

Define disaster recovery plans, including backup data storage locations, recovery time objectives (RTO), and recovery point objectives (RPO). Outline procedures for maintaining business continuity during disruptions.

# Types of ICT infrastructure

Traditional and cloud infrastructure are the two main types of IT infrastructure.

# Traditional infrastructure

The typical hardware and software components of a traditional IT infrastructure include facilities, data centres, servers, networking hardware, desktop PCs, and corporate application software solutions. This infrastructure arrangement typically requires more power, physical space, and financial resources than other infrastructure kinds. A conventional infrastructure is often built on-premises for use by a single organization or in a private environment.

# Cloud Infrastructure

The components and resources required for cloud computing are referred to as cloud infrastructure. End users, on the other hand, can connect to the infrastructure over the internet and utilize computing resources without having to install anything on their own due to virtualization. Virtualization connects physical servers that a service provider maintains in one or more locations. The system then splits and abstracts resources, like storage, so that users can access them practically everywhere there is an internet connection.

The term "public cloud" refers to cloud infrastructure that is frequently made available to the general public. You can develop your own private cloud employing resources that are completely dedicated to you. You may also establish a hybrid cloud by including some degree of workload portability, orchestration, and management across various clouds.

# Traditional vs Cloud IT Infrastructure

Traditional and cloud IT infrastructures differ in a number of ways.

7

# Automation

The way cloud and traditional IT infrastructure are handled is the most significant distinction. When you choose cloud hosting, you are effectively delegating all hardware and security concerns to the service provider. Traditional servers need in-house management of the infrastructure by an IT department.

# Costs

When compared to traditional IT infrastructure, cloud computing is more cost-effective. When you choose to cloud IT infrastructure, you only pay for what you utilize. Traditional IT infrastructure costs a lot of money to set up since you need to buy a lot of equipment and extra server space.

# Scalability

In comparison to traditional IT infrastructure, cloud hosting services give greater scalability. Cloud computing provides additional server resources and infinite storage space. Cloud servers can be scaled up or down based on your company's needs and business objectives. When it comes to traditional IT infrastructure, you usually only have a limited amount of resources, which means your system can't scale quickly. The only alternative is to purchase or rent a new server.

# Safety precautions of ICT Infrastructure

Ensuring the safety of ICT (Information and Communication Technology) infrastructure is crucial to protect both the technology assets and the people who work with them. Here are some safety precautions and best practices for ICT infrastructure:

# Physical Security:

Access Control: Limit physical access to data centers, server rooms, and networking equipment to authorized personnel only. Use access control systems, biometrics, and security badges.

Surveillance: Install security cameras to monitor and record access to critical infrastructure areas.

Environmental Controls: Implement environmental controls, such as fire suppression systems, temperature and humidity monitoring, to prevent equipment damage.

Cable Management: Keep cables organized and properly labeled to prevent tripping hazards and facilitate maintenance.

# Fire Safety:

Fire Suppression Systems: Install and regularly maintain fire suppression systems like sprinklers or gas-based extinguishing systems in server rooms and data centers.

Smoke Detectors: Use smoke detectors to provide early warning of fire incidents.

# Electrical Safety:

UPS (Uninterruptible Power Supply): Employ UPS systems to provide backup power during electrical outages and to safeguard against voltage fluctuations.

Surge Protectors: Use surge protectors and power conditioners to protect equipment from voltage spikes.

Grounding: Ensure proper grounding of all electrical equipment to prevent electrical shocks and fires.

# Environmental Controls:

8

Temperature and Humidity Monitoring: Maintain optimal temperature and humidity levels to prevent equipment overheating or damage.

Air Filtration: Use air filtration systems to reduce dust and particulate matter that can clog equipment and cause overheating.

# Data and Network Security:

Firewalls and Intrusion Detection Systems (IDS): Implement network security measures to protect against cyber threats and unauthorized access.

Data Encryption: Encrypt sensitive data to prevent unauthorized access in case of breaches.

Regular Backups: Perform regular data backups to prevent data loss due to hardware failure, cyberattacks, or accidents.

# Emergency Response:

Emergency Plans: Develop and regularly update emergency response plans that include procedures for evacuating personnel and protecting critical infrastructure during disasters.

Communication: Ensure that communication systems, such as emergency notification systems, are in place to alert staff during emergencies.

# Equipment Maintenance:

Regular Inspection: Schedule regular inspections of ICT equipment to identify and address potential issues before they become critical.

Vendor Support: Establish maintenance contracts with equipment vendors for timely repairs and updates.

# Labeling and Documentation:

Equipment Labeling: Label equipment and cables clearly to facilitate troubleshooting and maintenance.

Documentation: Maintain up-to-date documentation of infrastructure configurations, including network diagrams and equipment inventories.

# Training and Awareness:

Employee Training: Provide training to staff on safety procedures and the proper handling of ICT equipment.

Safety Culture: Promote a safety-conscious culture within the organization.

# Regulatory Compliance:

Ensure compliance with relevant safety and environmental regulations, such as those related to electrical safety and hazardous materials disposal.

# Physical Ergonomics:

Ensure that workstations and equipment are ergonomically designed to prevent repetitive strain injuries and musculoskeletal disorders among personnel.

# Disaster Recovery and Business Continuity:

Develop and test disaster recovery and business continuity plans to ensure ICT systems can be quickly restored in case of major disruptions.

9

By following these safety precautions and best practices, organizations can minimize risks to their ICT infrastructure and promote a safe working environment for their personnel.

# Documentation of Infrastructure assets and their operational and service status

In the modern digital age, ICT infrastructure is fundamental for organizations to deliver services and communicate efficiently. Proper documentation of ICT infrastructure assets and their operational and service status is crucial for ensuring smooth operations and strategic decision-making.

# Asset Inventory:

Begin by compiling a comprehensive inventory of all ICT assets, including servers, network equipment, software licenses, and end-user devices. Assign a unique identifier to each asset for easy tracking.

# Asset Classification:

Categorize assets based on their type, function, location, and criticality. This helps prioritize maintenance and upgrades.

# Asset Attributes:

Document detailed attributes for each asset, including make and model, specifications, purchase date, warranty information, and software versions. Keep records of asset depreciation and value for financial purposes.

# Operational Status:

Continuously monitor and document the operational status of ICT assets to ensure they are running optimally. Implement monitoring tools to proactively identify and address issues.

# Service Status:

Document the service status of ICT assets, including their role in delivering specific services or applications. Measure performance against Service Level Agreements (SLAs) to ensure service quality.

# Configuration Management:

Maintain records of hardware and software configurations, network topologies, and system settings. Use configuration management tools to track changes and maintain consistency.

# Software Licensing:

Keep a record of all software licenses, including purchase date, expiration date, and compliance status. Regularly audit software usage to ensure compliance with licensing agreements.

# Security Documentation:

Document security measures and protocols for each ICT asset. Record security incidents, vulnerabilities, and patch management activities.

# Change Management:

Document all changes to ICT infrastructure, including updates, upgrades, and configuration modifications. Maintain change logs to track the history of changes.

# Backup and Disaster Recovery:

10

Document backup strategies, schedules, and the location of backup data. Maintain a disaster recovery plan and document the recovery process.

# Capacity Planning:

Monitor resource utilization (CPU, memory, storage) and document capacity trends. Use this data to plan for resource upgrades or expansions.

# Vendor Relationships:

Keep records of vendor contracts, contact information, and support agreements. Document interactions with vendors regarding maintenance, upgrades, and issue resolution.

# Documentation Accessibility:

Ensure that documentation is easily accessible to relevant IT staff and stakeholders. Implement a secure document management system with role-based access controls.

# Compliance and Regulations:

Stay compliant with relevant data protection and cybersecurity regulations (e.g., GDPR, HIPAA). Document compliance efforts, audits, and assessments.

# Training and Knowledge Transfer:

Document training materials and procedures for IT staff. Ensure knowledge transfer and documentation updates when staff members change.

# Regular Audits and Reviews:

Conduct periodic audits and reviews of ICT documentation to verify accuracy and completeness. Adapt documentation practices to changing technology and security requirements.

# Disposal and Decommissioning:

Document the retirement and disposal process for outdated or decommissioned ICT assets, including data wiping and environmentally responsible disposal.

Proper documentation of ICT infrastructure assets and their operational and service status is vital for maintaining efficiency, security, and compliance in today's digital landscape. It supports informed decision-making and ensures that organizations can adapt to evolving technology needs.

11

TOPIC 2: EVALUATE PERFORMANCE OF ICT COMPONENTS

Evaluating the performance of Information and Communication Technology (ICT) components is crucial for ensuring that these systems meet the intended objectives, operate efficiently, and deliver value to organizations and individuals.

The evaluation of ICT components should be an ongoing process, considering technical, operational, and strategic aspects. Regular assessments and adjustments are essential to ensure that these components continue to meet the evolving needs of the organization and its users.

# Points to consider when evaluating the performance of ICT components Defining Objectives and Metrics:

Start by clearly defining the objectives and expected outcomes of the ICT components. What do you want to achieve with these technologies? This will guide the evaluation process.

# Reliability and Availability:

Assess how often the ICT components are available and operational. Downtime can have a significant impact on productivity and service delivery.

# Scalability:

Evaluate the ability of ICT components to handle increased workloads or user demands. Are they scalable to accommodate future growth?

# Speed and Responsiveness:

Measure the speed at which ICT components respond to user requests. Slow response times can lead to user frustration and reduced efficiency.

# Security and Data Protection:

12

Examine the security measures in place to protect data and infrastructure. Evaluate how well these components safeguard against unauthorized access, data breaches, and cyber threats.

# Cost Efficiency:

Determine the total cost of ownership for ICT components, considering both initial investment and ongoing operational costs. Evaluate if the components are cost-effective in achieving their objectives.

# User Experience:

Collect feedback from end-users to understand their experiences with the ICT components. This can include surveys, user interviews, and usability testing.

# Integration and Interoperability:

Assess how well the ICT components integrate with other systems and technologies within the organization. Compatibility and interoperability are vital for seamless operations.

# Compliance and Regulations:

Ensure that ICT components comply with relevant industry standards and legal regulations. Non-compliance can lead to legal and reputational risks.

# Performance Monitoring Tools:

Utilize performance monitoring tools and software to track key performance indicators (KPIs). These tools can provide real-time data and historical performance trends.

# Capacity Planning:

Perform capacity planning to anticipate future needs and prevent resource shortages. This involves forecasting resource requirements based on expected growth.

# Fault Tolerance and Redundancy:

Evaluate the level of fault tolerance and redundancy in the ICT components. Redundant systems can help ensure continued operations in the event of failures.

# Energy Efficiency:

Consider the energy consumption of ICT components, especially in the context of environmental sustainability. Evaluate options for reducing energy consumption.

# Upgrades and Maintenance:

Assess the ease of performing upgrades and maintenance on the ICT components. Minimizing downtime during maintenance is crucial.

# Performance Improvement Strategies:

Develop strategies for improving the performance of ICT components based on evaluation results. This may involve software optimization, hardware upgrades, or process enhancements.

# Regular Review and Benchmarking:

Continuously review and benchmark the performance of ICT components to identify areas for improvement and stay up to date with technological advancements.

13

# Alignment with Business Goals:

Ensure that the performance of ICT components aligns with the overall strategic goals of the organization.

# ICT system Troubleshooting

Troubleshooting is the process of detecting, isolating and repairing faults in a given system.

Troubleshooting is also a systematic process used to locate the cause of a fault in a computer system and correct the relevant hardware and software issues.

# Types of troubleshooting

Troubleshooting can be categorized into different types based on the nature of the problem, the domain it applies to, and the methods used.

Each type of troubleshooting requires specific skills, knowledge, and tools. The approach taken may vary depending on the domain and the complexity of the problem being addressed

Here are some common types of troubleshooting:

# Hardware Troubleshooting:

This type of troubleshooting focuses on identifying and resolving issues related to physical components such as computers, printers, routers, and other hardware devices. Common hardware troubleshooting tasks include diagnosing faulty components, replacing or repairing hardware, and addressing connectivity problems.

# Software Troubleshooting:

Software troubleshooting deals with problems related to computer programs, operating systems, and applications. It includes tasks like resolving software errors, compatibility issues, software crashes, and configuration problems.

# Network Troubleshooting:

Network troubleshooting involves identifying and fixing issues within computer networks. It includes diagnosing network connectivity problems, resolving network latency, ensuring proper routing, and addressing security concerns.

# System Troubleshooting:

System troubleshooting encompasses the examination of an entire computer system or infrastructure to pinpoint issues that affect its overall performance and reliability. This may involve a combination of hardware and software troubleshooting.

# Electrical Troubleshooting:

Electrical troubleshooting is relevant in industries and environments where electrical systems are prevalent. It involves identifying and resolving issues related to power distribution, circuits, and electrical equipment.

# Mechanical Troubleshooting:

Mechanical troubleshooting is typically employed in industries where machinery and mechanical systems are used. It focuses on identifying and resolving problems related to mechanical equipment, such as pumps, motors, and conveyor systems.

# Diagnostic Troubleshooting:

14

Diagnostic troubleshooting often relies on the use of diagnostic tools and software to identify issues. It involves running tests and diagnostics to pinpoint the root cause of problems.

# User Support Troubleshooting:

User support troubleshooting focuses on assisting end-users with technical problems they encounter. This may include resolving issues related to software, hardware, or user error.

# Remote Troubleshooting:

Remote troubleshooting is the process of identifying and resolving issues on a system or network from a remote location. This is particularly useful for IT support teams and remote assistance.

# Performance Troubleshooting:

Performance troubleshooting aims to address issues related to system performance and efficiency. It may involve optimizing hardware and software components to enhance overall performance.

# Security Troubleshooting:

Security troubleshooting focuses on identifying and mitigating security vulnerabilities and breaches. Tasks may include investigating security incidents, patching vulnerabilities, and enhancing security measures.

# Environmental Troubleshooting:

Environmental troubleshooting involves identifying and resolving issues related to the physical environment, such as temperature control, humidity, and ventilation, especially in data centers and server rooms.

# Process Troubleshooting:

Process troubleshooting is often used in manufacturing and industrial settings. It focuses on identifying and resolving issues in production processes to ensure efficiency and quality.

# Hybrid Troubleshooting:

In complex systems or scenarios, a combination of different troubleshooting methods may be necessary to address multifaceted problems. This could involve aspects of hardware, software, network, and more.

# Root Cause Analysis:

Root cause analysis is a systematic approach to identifying the underlying causes of recurring or critical issues. It delves deep into the problem to prevent it from reoccurring.

# Troubleshooting methods and steps

Note:

Troubleshooting is often a systematic and iterative process, and the approach may vary depending on the specific issue and the systems involved. It's important to remain methodical and well-documented in your troubleshooting efforts to effectively identify and resolve problems.

- Step 1. Identify the problem.

- Step 2. Establish a theory of probable cause.

- Step 3. Test the theory to determine the cause.

- Step 4. Establish a plan of action to resolve the problem and implement the solution.

- Step 5. Verify full system functionality and, if applicable, implement preventive measures.

- Step 6. Document findings, actions, and outcomes.

15

Step 1. Identify the problem

During this step, gather as much information as possible from the customer and from the computer. The table below lists some of the information to gather from the customer.

Step 1: Identify the Problem

  - Company name

  - Contact name

Customer information

- Address

- Phone number

Computer configuration

- Manufacturer and model

- Operating system

- Network environment

- Connection type

Problem description

- Open-ended questions

- Closed-ended questions

Error messages Beep sequences LEDs

POST

A simple way to understand the troubleshooting process is with a flow diagram of troubleshooting activity and tasks.

16

# Step 2: Establish a Theory of Probable Cause

First, create a list of the most common reasons for the error. Even if the customer thinks there is a major problem, start with the obvious issues before moving to more complex diagnoses, as outlined here:

- Check whether the device is powered off.

- Determine whether the power switch for an outlet is turned off.

- Check whether the surge protector is turned off.

- Ensure that there are no loose external cable connections.

- Check whether there is a non-bootable disk in the designated boot drive.

- Look for the incorrect boot order in the BIOS setup.

17

List the easiest or most obvious causes at the top. List the more complex causes at the bottom. If necessary, conduct internal (logs, journal) or external (internet) research based on the symptoms. The next steps of the troubleshooting process involve testing each possible cause.

# Step 3: Test the Theory to Determine the Cause

You can determine an exact cause by testing your theories of probable causes one at a time, starting with the quickest and easiest. Some common steps to determine the cause of the problem are as follows:

- Ensure that the device is powered on.

- Ensure that the power switch for an outlet is turned on.

- Ensure that the surge protector is turned on.

- Ensure that external cable connections are secure.

- Ensure that the designated boot drive is bootable.

- Verify the boot order in the BIOS setup.

Once the theory is confirmed, you can determine the steps to resolve the problem. As you become more experienced at troubleshooting computers, you will work through the steps in the process faster. For now, practice each step to better understand the troubleshooting process.

If you cannot determine the exact cause of a problem after testing all your theories, establish a new theory of probable cause and test it. If necessary, escalate the problem to a technician with more experience. Before you escalate, document each test that you tried.

# Step 4: Establish a Plan of Action to Resolve the Problem and Implement the Solution.

After you have determined the exact cause of the problem, establish a plan of action to resolve the problem and implement the solution. Sometimes quick procedures can correct the problem. If a quick procedure does correct the problem, verify full system functionality and, if applicable, implement preventive measures. If a quick procedure does not correct the problem, research the problem further and then return to Step 2 to establish a new theory of the probable cause.

# NOTE

Always consider corporate policies, procedures, and impacts before implementing any changes.

After you have established a plan of action, you should research possible solutions such as the following: Help desk repair logs

Other technicians Manufacturer FAQs

Technical websites News groups Computer manuals

Device manuals Online forums Internet search

Divide large problems into smaller problems that can be analyzed and solved individually. Prioritize solutions starting with the easiest and fastest to implement. Create a list of possible solutions and implement them one at a time. If you implement a possible solution and it does not correct the problem, reverse the action you just took and then try another solution. Continue this process until you have found the appropriate solution.

# Step 5: Verify Full Functionality and, if Applicable, Implement Preventive Measures

18

After the repairs to the computer have been completed, continue the troubleshooting process by verifying full system functionality and implementing the preventive measures needed, as outlined here:

- Reboot the computer.

- Ensure that multiple applications work properly.

- Verify network and Internet connections.

- Print a document from one application.

- Ensure that all attached devices work properly.

- Ensure that no error messages are received.

Verifying full system functionality confirms that you have solved the original problem and ensures that you have not created another problem while repairing the computer. Whenever possible, have the customer verify the solution and system functionality

# Step 6: Document Findings, Actions, and Outcomes.

After the repairs to the computer have been completed, finish the troubleshooting process with the customer. Explain the problem and the solution to the customer verbally and in writing. The steps to take when you have finished a repair are as follows:

- Discuss the solution implemented with the customer.

- Have the customer verify that the problem has been solved.

- Provide the customer with all paperwork.

- Document the steps taken to solve the problem in the work order and in the technician’s journal.

- Document any components used in the repair.

- Document the amount of time spent on resolving the problem.

Verify the solution with the customer. If the customer is available, demonstrate how the solution has corrected the computer problem. Have the customer test the solution and try to reproduce the problem. When the customer can verify that the problem has been resolved, you can complete the documentation for the repair in the work order and in your journal. Include the following information in the documentation:

Description of the problem Steps to resolve the problem Components used in the repair

# Common Problems and Solutions for PCs.

As a technician, you will run into technical problems in your day-to-day routine that need your attention. As the issues arise, take the time to better understand the causes of problems and work through possible fixes. Be sure to document all that you do. This section discusses several common PC problems and suggested solutions.

# PC Common Problems and Solutions.

Computer problems can be attributed to hardware, software, networks, or some combination of the three. You will resolve some types of problems more often than others.

# HARDWARE TROUBLESHHOTING

Some common hardware problems are as follows:

- Storage device: Storage device problems are often related to loose or incorrect cable connections, incorrect drive and media formats, and incorrect jumper and BIOS settings.

- Motherboard and internal components: Motherboard and internal component problems are often caused by incorrect or loose cables, failed components, incorrect drivers, and corrupted updates.

19

- Power supply: Power problems are often caused by a faulty power supply, loose connections, and inadequate wattage.

- CPU and memory: Processor and memory problems are often caused by faulty installations, incorrect BIOS settings, inadequate cooling and ventilation, and compatibility issues.

- Displays: Display problems are often caused by incorrect settings, loose connections, and incorrect or corrupted drivers.

# Common Problems and Solutions for Storage Devices.

Table 4-2 shows the probable causes and possible solutions for storage devices.

COPY NOTES ON Troubleshooting Process (4.2) _ Preventive Maintenance and Troubleshooting _ Cisco Press

Table 4-2 Common Problems and Solutions for Storage Devices

Table 4-3 Common Problems and Solutions for Motherboards and

Internal Components

able 4-5 Common Problems and Solutions for CPUs and Memory

Table 4-6 Common Problems and Solutions for Displays

Table 4-7 Advanced Problems and Solutions for Hardware

20

# Possible causes of ICT systems failure:

- Unstable power

- Malfunctioning

- Mechanical faults

- Environmental factors

- User factors

# Unstable Power:

This can occur due to a variety of reasons, including:

  - Power Surges: Sudden increases in electrical voltage can damage electronic components such as motherboards, processors, and hard drives. Power surges can result from lightning strikes, faulty power lines, or switching on/off heavy electrical equipment.

  - Power Outages: Complete power outages can lead to ICT system failures as they cause the system to shut down abruptly. This can result in data loss and potential corruption of system files.

  - Brownouts: A brownout is a drop in voltage that can cause ICT systems to malfunction or perform poorly. It can lead to data corruption and hardware damage over time.

  - Frequency Variations: Variations in the frequency of the power supply can affect the stability of ICT systems, leading to erratic behaviour and potential hardware damage.

To mitigate the effects of unstable power, organizations often use uninterruptible power supplies (UPS) and surge protectors to provide a stable power source and protect their ICT systems from voltage fluctuations.

21

# Malfunctioning:

ICT systems can experience failures due to internal issues or software-related problems. Possible causes of malfunctioning include:

    - Software Bugs: Software glitches or bugs can lead to system instability, crashes, or data corruption. These issues may arise from programming errors, compatibility problems, or inadequate testing.

    - Hardware Failures: Components like hard drives, memory modules, and CPUs can fail due to wear and tear, manufacturing defects, or overheating. These hardware failures can disrupt the normal functioning of the system.

    - Virus and Malware Infections: Malicious software can infect ICT systems, causing a range of issues, from data theft to system crashes. Malware can compromise security and disrupt normal operations.

    - User Errors: In some cases, system failures may result from user errors such as accidental deletion of critical files, misconfigurations, or improper software installation.

To prevent and address malfunctioning, organizations regularly update software, implement strong cybersecurity measures, and conduct routine maintenance and system monitoring.

# Mechanical Faults:

Mechanical faults can cause ICT system failures when the physical components of the system break down. Some potential causes include:

- Overheating: Excessive heat can lead to the failure of components like CPUs, graphics cards, and power supplies. Inadequate cooling or dust buildup can exacerbate this issue.

- Hard Drive Failures: Mechanical hard drives can experience failures due to read/write head crashes, platter damage, or motor issues. This can result in data loss and system unresponsiveness.

- Fan Failures: Cooling fans are essential for preventing overheating in ICT systems. When these fans malfunction or stop working, it can lead to temperature-related failures.

- Physical Damage: Accidents, drops, or exposure to environmental hazards can cause physical damage to ICT systems, affecting the integrity and functionality of the components.

Organizations can minimize the risk of mechanical faults by ensuring proper ventilation and cooling, conducting regular hardware inspections, and taking precautions to protect ICT systems from physical damage.

# Environmental factors:

Environmental factors that cause ICT system failure can be

  - Natural disasters

  - Dust

  - Ventilation

# Natural Disasters:

Natural disasters, such as earthquakes, floods, hurricanes, tornadoes, and wildfires, can have a profound impact on ICT systems. These events can lead to the following issues:

- Water Damage: Flooding can lead to severe water damage to ICT equipment, causing electrical shorts, corrosion, and irreversible hardware failures. Water damage can also lead to data loss and render systems inoperable.

- Power Outages: Natural disasters often result in power outages, which can lead to abrupt system shutdowns and data loss. Power surges and fluctuations can also occur during these events, potentially damaging hardware components.

22

- Physical Damage: Earthquakes, hurricanes, and tornadoes can physically damage data centers, server rooms, and ICT infrastructure. This damage can result in the destruction of equipment, cables, and infrastructure, rendering systems inoperable.

e. Fire Hazards: Wildfires and building fires can pose a significant threat to ICT systems. Fire can not only destroy hardware but also lead to the release of smoke and soot, which can infiltrate and damage sensitive components.

To mitigate the impact of natural disasters, organizations often implement disaster recovery and business continuity plans. This includes off-site data backups, redundant data centers in geographically diverse locations, and the use of fire-resistant and waterproof enclosures for critical ICT equipment.

# Dust:

Dust is a common environmental factor that can negatively affect ICT systems, particularly in areas with poor air quality. Dust can lead to several problems:

- Overheating: Accumulated dust can clog cooling fans and vents, impeding proper ventilation. This can cause ICT equipment to overheat, leading to hardware failures and reduced performance.

- Short Circuits: Dust particles can conduct electricity and create short circuits on printed circuit boards or connectors. These short circuits can damage components and disrupt system operation.

- Increased Maintenance: Dust accumulation necessitates more frequent cleaning and maintenance, increasing operational costs and downtime.

To prevent dust-related ICT system failures, organizations can employ air filtration and purification systems, implement regular cleaning schedules, and use dust-resistant server racks and enclosures.

# Ventilation:

Proper ventilation is crucial for maintaining the optimal operating conditions of ICT systems. Inadequate ventilation can lead to various issues:

- Overheating: Without sufficient airflow, ICT components, such as CPUs and graphics cards, can overheat, leading to thermal throttling, system instability, and eventual hardware failures.

- Reduced Lifespan: Overheating caused by poor ventilation can shorten the lifespan of components and result in a higher incidence of hardware failures.

- Performance Degradation: Inadequate ventilation can cause ICT systems to run at suboptimal performance levels, affecting productivity and efficiency.

Organizations can address ventilation-related issues by designing data centers and server rooms with proper airflow patterns, installing cooling systems, using fans and heat sinks, and maintaining clear pathways for air circulation.

# User factors :

User factors can contribute to ICT (Information and Communication Technology) system failures in various ways. While ICT systems are designed to be user-friendly, errors, negligence, and lack of understanding can lead to problems. Here are some user factors that can cause ICT system failures:

- Human error

- Lack of training and knowledge

- Negligence

- Resistance to change

23

- Unauthorized software or shadow IT

- Inadequate communication

# Human Error:

User-related errors are a common cause of ICT system failures. These errors can include:

- Data Entry Mistakes: Incorrect data input, such as typos or transposed numbers, can lead to inaccurate information in databases, resulting in incorrect outputs or decisions.

- Misconfigurations: Users may inadvertently change system settings or configurations, causing unintended consequences or making the system inoperable.

- Accidental Deletion: Users can accidentally delete critical files or data, which may be challenging to recover, leading to data loss and system disruptions.

- Unauthorized Access: Improperly managed user access permissions can allow unauthorized users to gain access to sensitive data, leading to security breaches.

# Lack of Training and Knowledge:

Users who lack proper training or understanding of the ICT systems they use can contribute to system failures:

  - Inefficient Use of Software: Users may not know how to use software efficiently, leading to suboptimal system performance.

  - Security Vulnerabilities: Users who are unaware of cybersecurity best practices may fall victim to phishing attacks or inadvertently introduce malware into the system.

# Negligence:

Negligence by users can lead to ICT system failures:

- Failure to Back Up Data: Users who neglect to back up critical data may face data loss in the event of hardware failure or data corruption.

- Poor Password Management: Weak or easily guessable passwords, as well as password sharing, can compromise system security.

# Resistance to Change:

Some users may resist adopting new technologies or system upgrades, leading to issues:

- Compatibility Problems: Older software or hardware may not be compatible with new system updates, causing compatibility issues or system instability.

- Reduced Productivity: Resistance to new systems or processes may result in slower adoption, hampering productivity.

# Unauthorized Software or Shadow IT:

Users sometimes install unauthorized software or use unapproved third-party services:

- Security Risks: These unauthorized applications may introduce vulnerabilities and malware into the system.

- Compliance Issues: Unauthorized software can lead to compliance violations and legal problems for the organization.

24

# Inadequate Communication:

Poor communication among users can contribute to system failures:

- Lack of Coordination: If users do not communicate effectively, they may inadvertently perform actions that conflict with or disrupt each other's work.

- Misunderstandings: Miscommunications can lead to errors in interpreting system requirements, resulting in incorrect system usage.

To mitigate user-related ICT system failures, organizations can invest in user training and education, enforce best practices for data management and security, implement strict access control, and encourage a culture of responsibility and accountability among users. Additionally, clear communication and collaboration among users and IT personnel can help prevent and address user-related issues effectively.

TOPIC 3.	DIAGNOSE AND FIX ICT RELATED PROBLEMS

Diagnostic tools -are used to test and diagnose equipment.

# Identify diagnostic and repair tools and their functions

Diagnostic and repair tools help technicians and IT professionals identify, troubleshoot, and fix issues in hardware, software, and network components

# Commonly used diagnostic and repair tools and their functions:

Operating System Diagnostic Tools:

- Event Viewer (Windows): It logs system and application events, allowing technicians to identify errors, warnings, and critical events.

- System Profiler (Linux): Provides detailed information about hardware and software configurations.

- Apple Diagnostics (Mac): Helps diagnose hardware issues on Mac computers.

# System Information Utilities:

25

- MSInfo32 (Windows): Provides detailed information about a Windows system's hardware, software, and components.

- lshw (Linux): Lists hardware details on a Linux system.

- System Information (Mac): Offers detailed system and hardware information on Mac computers.

# Hardware Diagnostic Tools:

- Memtest86: Checks RAM for errors and memory-related issues.

- SMART Monitoring Tools: Monitor and analyze the health of hard drives and SSDs.

- Hardware Diagnostic Suites (e.g., PC Doctor, HP Insight Diagnostics): Perform comprehensive hardware tests on various components.

# Network Diagnostic Tools:

- Ping: Tests network connectivity and measures latency between devices.

- Traceroute/Tracepath: Traces the route packets take through a network.

- Wireshark: Analyzes network traffic and helps diagnose network issues.

- Netstat: Displays active network connections, routing tables, and network interface statistics.

# Software Diagnostic Tools:

- Task Manager (Windows): Monitors and manages running processes, services, and resource usage.

- htop (Linux): Provides a dynamic overview of system processes and resource utilization.

- Activity Monitor (Mac): Offers information about running processes and system resources on Mac.

# Data Recovery Tools:

- Recuva: Recovers deleted files and data from storage devices.

- TestDisk: Recovers lost partitions and repairs file systems.

- PhotoRec: Specialized tool for recovering digital photos and media.

# Remote Desktop and Management Tools:

- Remote Desktop Protocol (RDP): Allows remote access to and control of a Windows-based computer.

- SSH (Secure Shell): Provides secure remote access to Unix-like systems.

- TeamViewer, AnyDesk, and other remote desktop software: Enable remote support and troubleshooting.

# Diagnostic Scripts and Command Line Utilities:

- Diagnostics Scripts (e.g., PowerShell and Bash scripts): Custom scripts for specific diagnostic and repair tasks.

- Command Line Utilities (e.g., ipconfig, ifconfig, and netsh): Used to troubleshoot network and system configuration issues.

# Antivirus and Anti-Malware Software:

- Tools like Windows Defender, Malwarebytes, and McAfee can scan and remove malware, viruses, and other security threats.

# Disk Cleanup and Optimization Tools:

- Tools like Disk Cleanup (Windows) and BleachBit (Linux) help clean up unnecessary files and optimize system performance.

26

# Commonly used ICT tools

A digital multimeter

# A digital multimeter

It tests the integrity of circuits and the quality of electricity in

computer components. A digital multimeter displays the information on an LCD or

LED.

# Tools and Techniques used to diagnose Hardware related problems

| Hardware problem | Diagnose | Tool |
| --- | --- | --- |
| No Power or Startup Issues: | Ensure the power source is working, check power cables and connections, and inspect the power supply unit (PSU). | Multimeter for testing power output, replacement power cables or adapters, and a spare PSU if available. |
| Overheating and Cooling Problems: | Monitor system temperatures, check for dust and debris in fans and heat sinks, and inspect thermal paste | Hardware monitoring software, compressed air for cleaning, and thermal paste for reapplication |
| Hardware Component | Identify the faulty component by testing or | Diagnostic tools (e.g., Memtest86+ for RAM testing), |

27

| Hardware problem | Diagnose | Tool |
| --- | --- | --- |
| Failures: | swapping hardware components. | spare hardware components for testing and replacement. |
| Data Storage Issues (HDD/SSD): | Use SMART diagnostics to check for drive health, and test for bad sectors or read/write errors | SMART monitoring tools (e.g., CrystalDiskInfo, GSmartControl), CHKDSK (Windows), fsck (Linux). |
| Peripheral Problems (e.g., Keyboard, Mouse, Monitor): | Check for loose connections, damaged cables, or driver issues. | Replacement cables, alternate peripherals, and driver updates. Connectivity and Port Issues: |
| Connectivity and Port Issues: | Test different ports, check for damaged connectors, and verify driver installations. | Different ports and cables, driver update software |
| Random Reboots or Freezes: | Monitor system temperatures, check for overheating, and inspect for dust and loose connections. | Hardware monitoring software, temperature sensors, and system logs. |
| Peripheral Driver Issues: | Check for driver conflicts or outdated drivers, reinstall or update drivers. | Device Manager (Windows), lsusb (Linux), System Information (Mac), and manufacturer websites for driver downloads |
| BIOS/UEFI Problems: | Check BIOS/UEFI settings for correct configurations and version updates. | BIOS/UEFI setup utility, manufacturer's website for firmware updates. |
| Graphics Card Issues: | Test with different monitors, check for loose connections, and update graphics drivers. | Spare monitor or graphics card (if available), graphics driver update software. |
| Expansion Card Problems (e.g., sound, network cards): | Check for loose connections, ensure drivers are up to date, and test with different expansion slots. | Device Manager (Windows), lspci (Linux), manufacturer's website for driver updates. |
| Motherboard or Power Supply Unit (PSU) Problems: | Inspect for physical damage, test with a known working PSU, and check for motherboard capacitors bulging or leaking. | Multimeter for PSU testing, spare PSU for testing, visual inspection for motherboard issues. |

# Tools and Techniques used to diagnose software related problems

| Software problem | Diagnose | Tool |
| --- | --- | --- |
| Application Crashes and Freezes: | Examine error messages, logs, and event viewer entries to identify the problematic application or component. | Event Viewer (Windows), logs (Linux), and Console (Mac) for error messages. |
| Operating System Boot Issues: | Analyze the boot sequence, identify potential errors or corruption in the boot files, and inspect hardware connections. | Windows Recovery Environment (Windows), recovery mode (Linux), macOS Recovery (Mac). |
| Driver Problems: | Check for driver conflicts, outdated or incompatible drivers, and examine the Device Manager (Windows) or system logs. | Device Manager (Windows), lsusb/lspci (Linux), System Information (Mac), manufacturer websites for driver updates. |
| Software Compatibility Issues: | Identify conflicting software, incompatible versions, or dependencies, and verify system requirements. | System logs, error messages, and software version information. |
| Malware and Virus Infections: | Use antivirus and anti-malware software to scan and remove malicious software. | Antivirus and anti-malware programs (e.g., Windows Defender, Malwarebytes). |
| File and Data Corruption: | Use file integrity checks, like the System File Checker (SFC) in Windows, to identify and repair corrupted files | SFC (Windows), fsck (Linux), Disk Utility (Mac). |

28

| Software problem | Diagnose | Tool |
| --- | --- | --- |
| Network Connectivity Issues: | Inspect network settings, check cables and hardware, and perform network tests (e.g., ping, traceroute). | Command-line utilities (ping, tracert/traceroute), network settings, and hardware diagnostics. |
| Slow System Performance: | Analyze system resource usage, check for resource-hungry applications, and optimize startup programs. | Task Manager (Windows), htop (Linux), Activity Monitor (Mac), and startup management tools. |
| Software Update Problems: | Investigate update logs and errors, ensure a stable internet connection, and check for update settings. | Windows Update Troubleshooter (Windows), package manager logs (Linux), App Store (Mac). |
| Missing or Deleted Files: | Search for files in trash or recycling bins, use file recovery software, and check backup solutions. | File recovery software (e.g., Recuva, TestDisk, PhotoRec), backup systems. |
| User Account and Password Issues: | Check for forgotten or locked user accounts, password issues, and account settings. | User account management tools (e.g., User Accounts in Windows, passwd in Linux). |
| Software Configuration Errors: | Examine application settings, configurations, and logs to identify and correct errors. | Application settings, configuration files, and error logs. |
| Browser Issues: | Clear browser cache and cookies, disable extensions, update the browser, and reset browser settings. | Browser settings and options, browser extensions, and built-in troubleshooting tools. |
| Application-specific Problems: | Consult application-specific documentation, forums, or support resources for troubleshooting steps. | Application-specific troubleshooting guides and forums. |

For more complex or persistent software-related problems, it may be necessary to seek assistance from software developers, IT support, or online communities. Thoroughly documenting error messages, symptoms, and the steps that led to the issue can be valuable when seeking help or trying to resolve the problem independently.

# Tools and Techniques used to diagnose internet/network related problems

| Internet /network problem | Diagnose | Tool |
| --- | --- | --- |
| No Internet Connectivity: | Check network cables, power cycle the modem and router, and verify your device's Wi-Fi or Ethernet connection. | Network status icons on the device. |
| Slow Internet Speed: | Run a speed test, check for concurrent network activity, and monitor for background downloads or streaming. | Online speed testing services (e.g., Ookla Speedtest), task manager for monitoring network usage |
| Wi-Fi Connectivity Issues: | Verify Wi-Fi signal strength, reset the router, and check for interference from other devices or networks. | Wi-Fi analyzer apps, router reboot buttons, and network diagnostic tools on devices. |
| Network Configuration Problems: | Check IP settings, subnet masks, and default gateways to ensure they are correctly configured. | Command-line utilities (ipconfig/ifconfig), network settings in device settings. |
| DNS Resolution Issues: | Test DNS resolution with the "ping" command, try different DNS servers, and clear the DNS cache. | Command-line utilities (ping, nslookup), changing DNS server settings in device or router. |
| Port Forwarding and NAT Issues: | Verify port forwarding settings for specific applications or devices, check for double NAT. | Router web interface, online port checking tools, and NAT type tests for gaming consoles. |
| Firewall and Security Software Blocking: | Check firewall rules and settings, inspect antivirus and security software for | Firewall settings, security software settings, and application-specific settings. |

29

| Internet /network problem | Diagnose | Tool |
| --- | --- | --- |
|  | network blocking. |  |
| Network Hardware Problems: | Inspect network hardware for physical damage, overheating, and firmware updates. | Physical inspection, firmware updates from manufacturer websites. |
| Network Packet Loss: | Use network monitoring tools to identify packet loss and latency issues. | Command-line utilities (ping, traceroute), network monitoring software (e.g., Wireshark). |
| VPN Connection Issues: | Verify VPN settings, check for firewall or antivirus interference, and test the VPN on different networks. | VPN settings, firewall settings, and VPN provider support resources. |
| Intermittent Connection Problems: | Monitor the network for dropouts, check cables and connections, and replace faulty network hardware. | Network monitoring tools, cable and connection inspection, replacement hardware. |
| Router or Modem Rebooting or Overheating: | Ensure proper ventilation, reboot the router, and check for firmware updates. | Router reboot buttons, firmware updates from manufacturer websites. |
| MAC Address Filtering: | Check the router's MAC address filtering settings, and ensure the device's MAC address is allowed. | Router settings and device MAC address information. |
| ISP-related Issues: | Contact your Internet Service Provider (ISP) to inquire about service disruptions or line issues. | Contact ISP support and service status pages on the ISP's website. |
| Outage in Your Area: | Check your ISP's outage notifications or contact them for information about network disruptions. | ISP support and service status updates on the ISP's website. |

Tools and Techniques used to diagnose User related related problems

| User related related problems | Diagnose | Tool |
| --- | --- | --- |
| Forgotten Passwords: | Verify the user's identity and reset the password. | Password reset tools, security questions, or identity verification processes. |
| Locked Out of Account: | Identify the reason for the lockout (e.g., multiple failed login attempts) and unlock the account. | Account unlock tools and procedures, security questions. |
| Permission Issues: | Determine whether the user has the appropriate permissions to access a file, folder, or resource. | User account management tools, file or resource permissions settings |
| Software Usage Problems: | Assist the user in understanding and resolving issues related to software usage. | Screen sharing or remote desktop tools, user training and documentation. |
| Email Configuration Issues: | Verify email settings, check for typos, and assist with configuring email clients. | Email client settings, remote support, email server documentation. |
| Account Deactivation or Termination: | Ensure compliance with company policies, backup or transfer necessary data, and inform the user. | User account management tools, data backup and migration tools. |

30

| User related related problems | Diagnose | Tool |
| --- | --- | --- |
| Data Loss or Deletion: | Assess the extent of data loss, check for data backups, and attempt data recovery if possible. | Data recovery software, backups, and recovery tools. |
| Device and Hardware Setup Problems: | Assist users in setting up and configuring their devices and peripherals. | Device manuals, remote support, and setup guides. |
| Account Compromises and Security Breaches | Investigate the extent of the breach, change passwords, and implement additional security measures. | Security incident response procedures, password change tools, and two-factor authentication. |
| Software Installation Issues: | Troubleshoot software installation problems, check system requirements, and resolve conflicts. | Installation logs, error messages, and software support resources. |
| User Training and Education: | Assess the user's knowledge and provide training or resources to improve their ICT skills. | Training materials, online tutorials, and user education programs. |
| Browser or Website Problems: | Help users with browser settings, extensions, and navigating websites. | Browser settings, browser extension management, and web support resources. |
| Printers and Peripheral Issues: | Assist users with printer setup, driver installations, and resolving peripheral problems. | Printer manuals, driver downloads, and remote support |
| Remote Access and VPN Issues: | Troubleshoot remote access problems and VPN connectivity issues. | VPN client settings, remote access guides, and support from IT teams. |
| Communication Problems (e.g., VoIP or Video Conferencing): | Help users with communication tools, check settings, and test connectivity. | Communication software settings, network monitoring tools, and user training resources. |

BEGIN FROM HERE!!!!!

# Role of ICT Policies in organizations

ICT (Information and Communication Technology) policies play a vital role in organizations, as they provide a framework for the management, usage, and security of technology resources. These policies help ensure that technology is used effectively, responsibly, and in alignment with an organization's goals and legal requirements. Here are some key roles and benefits of ICT policies in organizations:

Guidance and Standardization: ICT policies establish guidelines and standards for the acquisition, deployment, and use of technology resources. This ensures consistency and uniformity in technology implementations across the organization.

Security and Data Protection: ICT policies outline security measures and best practices to protect the organization's data, information systems, and networks. They address issues such as data encryption, access controls, and data backup procedures.

Compliance and Legal Requirements: These policies help organizations adhere to legal and regulatory requirements, including data protection laws (e.g., GDPR, HIPAA) and industry-specific regulations. Non- compliance can lead to legal consequences and financial penalties.

Data and Privacy Protection: ICT policies define how sensitive information and personal data are handled, stored, and shared within the organization. They ensure that data privacy is respected and maintained.

31

Resource Allocation: These policies help in managing technology resources efficiently, such as determining who has access to which software, hardware, and network resources. This prevents resource wastage and ensures that technology investments are used effectively.

Risk Management: ICT policies identify potential technology-related risks and provide procedures for mitigating them. This includes disaster recovery plans, cybersecurity incident response plans, and business continuity strategies.

User Accountability: These policies clarify the responsibilities and expectations of employees, contractors, and other users in terms of technology usage. They help hold individuals accountable for their actions and ensure responsible use of ICT resources.

Network Management: Policies related to network usage, bandwidth allocation, and traffic management ensure that network resources are allocated appropriately and that critical applications have priority.

Software Licensing and Usage: Policies define how software is acquired, licensed, and used within the organization. This helps prevent software piracy, licensing violations, and costly legal issues.

Employee Training and Awareness: ICT policies may include provisions for employee training and awareness programs to ensure that staff is well-informed about technology best practices and security measures.

Incident Response: These policies outline procedures for reporting and responding to ICT incidents, such as security breaches, data breaches, and system failures.

Resource Conservation: ICT policies encourage the efficient use of resources to reduce energy consumption, waste, and environmental impact. This aligns with sustainability and corporate social responsibility objectives.

Accountability and Governance: ICT policies often define roles and responsibilities of technology governance bodies within the organization, ensuring accountability and oversight of technology initiatives.

Change Management: Policies guide how changes to technology systems and infrastructure are planned, tested, and implemented to minimize disruptions and errors.

Vendor and Supplier Relations: ICT policies can address how the organization interacts with technology vendors and suppliers, including procurement processes and contract management.

32

TOPIC 4: PERFORM USER TRAINING

Performing user training is a crucial aspect of ICT (Information and Communication Technology) system support. User training ensures that individuals who use ICT systems have the knowledge and skills required to operate, maintain, and troubleshoot these systems effectively.

# Importance/Significance of performing user training in ICT system support:

- Enhancing User Competence:

User training aims to improve the competence of individuals using ICT systems. It empowers users to navigate the system, use software applications, and perform tasks efficiently. When users are well-trained, they are less likely to make errors, leading to a reduction in system failures and increased productivity.

# Increasing System Adoption:

Proper training encourages users to embrace new technologies and system upgrades. Users who understand the benefits and functionalities of ICT systems are more likely to use them effectively.

# Reducing Support Requests:

Adequate user training can help minimize the number of support requests and inquiries. Users who are well-trained can resolve common issues independently, reducing the burden on IT support teams.

# Enhancing Security Awareness:

User training is essential for promoting cybersecurity awareness. It educates users on best practices for data security, password management, and how to recognize and avoid security threats.

# Enhanced Productivity and Efficiency:

Well-trained users are more efficient in performing their tasks, as they understand how to use ICT systems and software effectively. This increased efficiency leads to better productivity, allowing users to complete tasks in less time.

# Reduced User Errors:

Training helps users avoid common mistakes and errors, such as data entry mistakes, misconfigurations, and accidental deletions. By reducing errors, user training contributes to a decrease in system downtime and data loss.

# Effective System Utilization:

Proper training enables users to fully utilize the capabilities of ICT systems, including software applications and features. Users can take advantage of the full range of functionalities, leading to better results and decision-making.

# Increased User Confidence:

Users who are well-trained feel more confident in using ICT systems. Confidence fosters a positive attitude towards technology, reducing anxiety and resistance to change.

# Enhanced Security Awareness:

33

User training educates individuals about cybersecurity best practices, teaching them how to recognize and respond to security threats. Informed users are less likely to fall victim to phishing, malware, and other cyberattacks, improving overall system security.

# Facilitated System Upgrades and Adaptation:

Users who receive training are more likely to embrace new technologies and system upgrades. This smooth transition to new systems or software versions helps organizations stay current and competitive.

# Reduced IT Support Burden:

A well-trained user base is less reliant on IT support for common issues, reducing the volume of support requests. IT personnel can focus on more complex and critical tasks, improving overall support efficiency.

# Improved Data Management:

Training helps users understand data management best practices, such as data backup and storage. Users are better equipped to protect and manage critical data, reducing the risk of data loss.

# Customized Training for Specific Needs:

Training programs can be tailored to the specific needs of different user groups within an organization. This customization ensures that training aligns with the roles and responsibilities of users.

# Continuous Learning and Adaptation:

ICT systems and technologies are constantly evolving. Ongoing training allows users to adapt to new features and improvements. It ensures that users remain up-to-date with the latest advancements in technology.

# Compliance and Regulations:

User training is essential for organizations to comply with industry regulations and data protection laws. In some sectors, compliance may require employees to undergo specific training related to privacy and security.

# Positive Impact on Organizational Culture:

A culture of learning and continuous improvement is fostered through user training. It demonstrates an organization's commitment to investing in the skills and development of its employees.

# Key Considerations for User Training:

- Customization: Training programs should be tailored to the specific needs and skill levels of the users. Different user groups may require different training approaches.

- Hands-On Learning: Practical, hands-on training sessions are often more effective than theoretical instruction alone. Ongoing Training: ICT systems and software are continually evolving. Regular, ongoing training ensures that users stay up to date with new features and improvements.

- Multimedia and Resources: Incorporating various training resources, including videos, manuals, and online tutorials, can cater to different learning styles.

- Feedback and Evaluation: Regular feedback and assessment help identify areas where users may need additional training or support.

- Documentation: Training materials and documentation should be readily available for users to reference as needed.

34

- Support Channels: Users should be aware of how to access support resources, such as help desks or online forums, in case they encounter issues.

# Challenges in User Training:

- Resistance to Change: Some users may be resistant to training or change, which can be a challenge for trainers. Time and Resource Constraints: Organizations may face limitations in terms of time, budget, and resources for training programs.

- Diverse User Base: Large organizations may have users with varying levels of IT literacy, making it challenging to create a one-size-fits-all training program.

- Measuring the Effectiveness of User Training: The success of user training can be evaluated through user feedback, proficiency tests, and the reduction in support requests. Continuously monitor and update training programs to address evolving user needs and technology changes.

# Step by step guide in implementing end-user training plan

Implementing an end-user training plan is crucial to ensure that users have the knowledge and skills they need to effectively and efficiently use ICT (Information and Communication Technology) systems.

# Needs Assessment:

Identify the specific training needs of your end users. Consider their roles, skill levels, and the systems or software they will be using. Gather feedback from users to understand their concerns, challenges, and learning preferences.

# Set Clear Objectives:

Define clear and measurable training objectives. What should users be able to do after the training? Objectives should be aligned with the organization's goals and the users' roles.

# Develop Training Materials:

Create training materials, including user manuals, video tutorials, presentations, and hands-on exercises. Ensure that the materials are easy to understand and comprehensive.

# Choose Training Methods:

Select the most appropriate training methods based on your users' needs. Common methods include in-person training sessions, online courses, webinars, and self-paced e-learning modules. Consider a blended approach that combines various methods for optimal learning.

# Schedule Training Sessions:

Develop a training schedule that accommodates users' availability and work commitments. If necessary, offer multiple sessions to cater to different user groups.

# Select Trainers or Instructors:

Identify qualified trainers or instructors who are knowledgeable about the ICT systems and can effectively deliver the training. Ensure that trainers have strong communication and teaching skills.

# Customize Content:

35

Tailor training content to meet the specific needs and skill levels of different user groups. Customize examples and exercises that are relevant to users' job roles.

# Deliver Training:

Conduct the training sessions according to the established schedule and delivery methods. Ensure that the training is interactive, engaging, and allows users to practice what they've learned. Encourage questions and discussion to address any concerns or confusion.

# Assess and Evaluate:

Assess user learning through quizzes, practical exercises, or other forms of evaluation. Collect user feedback to gauge the effectiveness of the training and identify areas for improvement.

# Provide Resources:

Offer users access to training materials and resources for reference and continued learning. Create a central repository for documentation and tutorials.

# Support and Follow-Up:

Establish a support system for users who may have questions or encounter issues after the training. Schedule follow- up sessions or periodic assessments to reinforce learning and ensure ongoing skill development.

# Monitor Progress:

Continuously monitor the progress of users in applying their newly acquired skills. Identify and address any emerging training needs or gaps.

# Adapt and Improve:

Be prepared to adapt the training plan based on user feedback and evolving technology. Continuously improve the training program to align with changing organizational needs.

# Document Training Records:

Maintain records of training sessions, including attendance, assessment results, and user feedback. This documentation helps track progress and compliance with training requirements.

# Promote a Culture of Learning:

Encourage a culture of continuous learning within the organization. Recognize and reward employees who demonstrate proficiency and embrace training opportunities.

# Compliance and Certification:

In some industries, compliance regulations may require certification or documentation of training. Ensure that your training program complies with relevant regulations.

# Budget and Resources:

Allocate the necessary budget and resources for the training plan, including instructor fees, materials, and training facilities or software.

# Communication and Marketing:

36

Communicate the training plan to all users, providing clear information about the objectives, benefits, and schedule. Use marketing strategies to generate interest and enthusiasm among users.

37